Weekly Report

5th Week

In this week, the following topic has been handled:

Design Optimization topic is very vast and widely used technology for the improvement of the Structures while pre-development process. Design Optimization refers to the process of finding the optimal structural design by changing the design parameters which are used and can be modified.

This search is guided by the need to satisfy operating limits for the response of the system. During this simulation, the design is guided by limits that are imposed on the response of the structure and by further limits on the parameters of the structure that can be assumed.

When performing an optimization task in Nastran, only those analyses and design parameters can be accessed that were explicitly included in the design of the capability, and that are applicable. While reading this paragraph, it may come to mind that, what the reason to use the design optimization though, through the Structural analysis, this task can also be performed. The main aim of design optimization is to automate the design cycle with a rational, mathematical approach to yield design improvement and additionally producing more effective designs having maximum spaces of safety.

In MSC Nastran, design optimization can be operated by using the solution sequence 200 in short SOL200. While performing this task, additional input cards will be included in the analysis input card for Nastran. The new Cards are design objectives, design variables, design constraints, side constraints, and many more other cards that can be implemented according to the need of the User. The figure \ref{fig:desop} illustrates the idea behind the design optimization where the model will be analyzed and with the DO parameters also the design optimization task will be run. The result will be then checked with the design constraints whether these restrictions have been violated or not. If the desire requirements have been achieved or arrived at the critical point, the loop will be terminated otherwise the execution of the optimization task will be continued.

Additionally this week, I have also conducted some Analysis of the plate made up of composite materials. Furthermore, Some SOL 200 examples have been also run in MSC Nastran and the visualization of the result in Patran looks like as in the figure \ref{fig:plate2} and \ref{fig:pcomp2}.

Geometry

Boundary Conditions

Loading

Materials

Analysis

Design Optimization (DO)

Model

Results

Requirements meet

Optimal Design

no

Yes

Model

(.bdf file)

Analysis

(SOL200)

Requirements meet

Design Optimization

Optimal Design

no

Yes

MatLab (Opt. cards)

ModGen(Input files)

Geometry

Boundary Conditions

Loading

Materials

Properties